| Company Project number Serial number Material number Material description Date Issued by Revision |  |  | Tetra Pak Ord_T5844151405 T5844151405 3078300-0100 Tetra Therm Aseptic Flex 2018-04-13 Niklas Persson 0, 2018-07-02 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item No. | Label | No |  | Article No. | Denomination | Type | Doc No | Note | Rev. |
| 0 TA Flex |  |  | 3078300-0100 |  |  |  |  |  |  |
| 1 | _ | 1 | L | 3003921-0100 | Booster module |  | No doc. |  |  |
| 1 | _ | 1 | L | 2895143-0100 | Main module |  | No doc. |  |  |
| 1 | CR49.2 | 1 | L | 6-31801 5046 1 | Steam trap | FT14-10 R-L, DN20 G3/4" PN16 | 1211779-0101 |  |  |
| 1 | CR50.2 | 1 | L | 6-31801 5046 1 | Steam trap | FT14-10 R-L, DN20 G3/4" PN16 | 1211779-0101 |  |  |
| 1 | DV11 | 1 | L | 90513-8306 | Air vent valve SS | With ball valve DN15 PN10, no fitting Type AB050/R002 | 1212676-0101 |  |  |
| 1 | F49 | 1 | L | 6-31801 5609 2 | Strainer | Fig 12 ISO228/1-G3/4 PN25 SS screen 0.8mm SG Iron | 1248674-0101 |  |  |
| 1 | F50 | 1 | L | 6-31801 5609 2 | Strainer | Fig 12 ISO228/1-G3/4 PN25 SS screen 0.8mm SG Iron | 1248674-0101 |  |  |
| 1 | FT10 | 1 | L | 90515-8866 | Flow indicator | H250/RR/M40 2" SMS 1145 PN16 ESK 2500-25000l/h, 4- 20mA | 1229491-0101 |  |  |
| 1 | GS29 | 1 | L | 344254-0150 | Sensor cable |  | No doc. |  |  |
| 1 | GS29 | 1 | L | 90510-0270 | Proximity switch | M18x1 L=70mm, SS, IP69K, M12-conn. 10-36VDC, 3-w.PNP/2 w.PNP/NPN Sensing range 0-8mm | 1241250-0101 |  |  |
| 1 | GS30 | 1 | L | 344254-0150 | Sensor cable |  | No doc. |  |  |
| 1 | GS30 | 1 | L | 90510-0270 | Proximity switch | M18x1 L=70mm, SS, IP69K, M12-conn. 10-36VDC, 3-w.PNP/2 w.PNP/NPN Sensing range 0-8mm | 1241250-0101 |  |  |
| 1 | GS31 | 1 | L | 90510-0270 | Proximity switch | M18x1 L=70mm, SS, IP69K, M12-conn. 10-36VDC, 3-w.PNP/2 w.PNP/NPN Sensing range 0-8mm | 1241250-0101 |  |  |
| 1 | GS31 | 1 | L | 344254-0150 | Sensor cable |  | No doc. |  |  |
| 1 | GS32 | 1 | L | 344254-0150 | Sensor cable |  | No doc. |  |  |
| 1 | GS32 | 1 | L | 90510-0270 | Proximity switch | M18x1 L=70mm, SS, IP69K, M12-conn. 10-36VDC, 3-w.PNP/2 w.PNP/NPN Sensing range 0-8mm | 1241250-0101 |  |  |
| 1 | GS33 | 1 | L | 344254-0150 | Sensor cable |  | No doc. |  |  |
| 1 | GS33 | 1 | L | 90510-0270 | Proximity switch | M18x1 L=70mm, SS, IP69K, M12-conn. 10-36VDC, 3-w.PNP/2 w.PNP/NPN Sensing range 0-8mm | 1241250-0101 |  |  |
| 1 | GS34 | 1 | L | 90510-0270 | Proximity switch | M18x1 L=70mm, SS, IP69K, M12-conn. 10-36VDC, 3-w.PNP/2 w.PNP/NPN Sensing range 0-8mm | 1241250-0101 |  |  |
| 1 | GS34 | 1 | L | 344254-0150 | Sensor cable |  | No doc. |  |  |
| 1 | GS35 | 1 | L | 90510-0270 | Proximity switch | M18x1 L=70mm, SS, IP69K, M12-conn. 10-36VDC, 3-w.PNP/2 w.PNP/NPN Sensing range 0-8mm | 1241250-0101 |  |  |
| 1 | GS35 | 1 | L | 344254-0150 | Sensor cable |  | No doc. |  |  |
| 1 | GS36 | 1 | L | 90510-0270 | Proximity switch | M18x1 L=70mm, SS, IP69K, M12-conn. 10-36VDC, 3-w.PNP/2 w.PNP/NPN Sensing range 0-8mm | 1241250-0101 |  |  |
| 1 | GS36 | 1 | L | 344254-0150 | Sensor cable |  | No doc. |  |  |

| Item No. | Label | No |  | Article No. | Denomination | Type | Doc No | Note | Rev. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | HE1 | 1 | L | 2827571-0100 | Tubular Heat Exchanger C85 |  | 1223855-0101 |  |  |
| 1 | M12 | 1 | L | 6-34144 6932 2 | CIP feed unit | With 320 l plastic tank Diaphragm pump TR20PTT-7SV, air reg. According to drawing: 34144-6932 Rev. 02 | 1222742-0101 |  |  |
| 1 | M13 | 1 | L | 6-34144 6932 2 | CIP feed unit | With 320 l plastic tank Diaphragm pump TR20PTT-7SV, air reg. According to drawing: 34144-6932 Rev. 02 | 1222742-0101 |  |  |
| 1 | PI08 | 1 | L | 90522-0142 | Pressure gauge | G3/8" A 0-25bar v. 233.50.100 lower mount | 1212430-0101 |  |  |
| 1 | PT01 | 1 | L | 90515-5472 | Pressure transmitter | PMP55 SMS38.0 0-37.5bar(g) 4-20mA , F15 SS house Cable gland M20x1.5 IP66/68 | 1229086-0101 |  |  |
| 1 | PT60 | 1 | L | 90515-5472 | Pressure transmitter | PMP55 SMS38.0 0-37.5bar(g) 4-20mA , F15 SS house Cable gland M20x1.5 IP66/68 | 1229086-0101 |  |  |
| 1 | PT705 | 1 | L | 90521-2993 | Pressure transmitter | PMP55 Varivent N 0-40bar(g) 4-20mA HART, F15 SS house w/o display Gland M20, IP66/68 NEMA4X/6P | 1229086-0101 |  |  |
| 1 | Tool | 1 | L | 6-9613 16 0901 | Tool | For Bonnet bushing | No doc. |  |  |
| 1 | Tool | 1 | L | 6-69696 00 | Spanner | SMS 25.0-76.1 / DIN 20-65 | No doc. |  |  |
| 1 | Tool | 1 | L | 6-9613 17 2901 | Mounting Tool | For Unique SSV Elastomer Plug Seal 1" /38mm/DN40 | No doc. |  |  |
| 1 | TT10 | 1 | L | 90510-5633 | Temperature transmitter | TPT12 4-20mA Measuring range 0-160 deg. C | 3283902-0101 |  |  |
| 1 | TT10 | 1 | L | 344254-0150 | Sensor cable |  | No doc. |  |  |
| 1 | TT42 REF | 1 | T |  | Plug for PT100 |  |  |  |  |
| 1 | TT42 | 1 | L | 90510-5633 | Temperature transmitter | TPT12 4-20mA Measuring range 0-160 deg. C | 3283902-0101 |  |  |
| 1 | TT42 | 1 | L | 344254-0150 | Sensor cable |  | No doc. |  |  |
| 1 | TT44 REF | 1 | T |  | Plug for PT100 |  |  |  |  |
| 1 | TT44 | 1 | L | 344254-0150 | Sensor cable |  | No doc. |  |  |
| 1 | TT44 | 1 | L | 90510-5633 | Temperature transmitter | TPT12 4-20mA Measuring range 0-160 deg. C | 3283902-0101 |  |  |
| 1 | TT45 | 1 | L | 90510-5633 | Temperature transmitter | TPT12 4-20mA Measuring range 0-160 deg. C | 3283902-0101 |  |  |
| 1 | TT45 | 1 | L | 344254-0150 | Sensor cable |  | No doc. |  |  |
| 1 | TT49 | 1 | L | 344254-0150 | Sensor cable |  | No doc. |  |  |
| 1 | TT49 | 1 | L | 90510-5633 | Temperature transmitter | TPT12 4-20mA Measuring range 0-160 deg. C | 3283902-0101 |  |  |
| 1 | TT50 | 1 | L | 344254-0150 | Sensor cable |  | No doc. |  |  |
| 1 | TT50 | 1 | L | 90510-5633 | Temperature transmitter | TPT12 4-20mA Measuring range 0-160 deg. C | 3283902-0101 |  |  |
| 1 | TT742 | 1 | L | 90525-6058 | Thermometer | DART-50-350F L=51/2 Cl2 230VAC 25ft with calibration certificate | 1214474-0101 |  |  |
| 1 | V22.1 | 1 | L | 6-9613 36 3185 | Single seat valve | SSV-W-51-210-NO-EPDM TP Change Over Maintainable actuator | 1221122-0101 |  |  |
| 1 | V22.2 | 1 | L | 6-9613 36 3185 | Single seat valve | SSV-W-51-210-NO-EPDM TP Change Over Maintainable actuator | 1221122-0101 |  |  |
| 0 Main Module 2895143-0100 |  |  |  |  |  |  |  |  |  |
| 1 | BTD01 | 1 | L | 6-31542 1679 1 | Balance tank | BTD 300 with Scaba Agitator Acc to drawing:31542-1679 | 1218895-0101 |  |  |
| 1 | BTD02 | 1 | L | 6-31542 1329 1 | CIP Header Batch | CIP HB tank 125L SMS Acc to dwg:31542-1329 Rev04 | 3281944-0101 |  |  |
| 1 | CP01 | 1 | L | 2565638-0100 | CCP | TA Flex 10/100 | 1228796-0101 |  |  |
| 1 | CR41 | 1 | L | 6-31801 9809 1 | Steam trap | MST21 G1/2" NTS-fill PN25 | 1212764-0101 |  |  |
| 1 | CR44 | 1 | L | 90511-6240 | Steam trap | DN25 FT43-10TV PN16 Flanged | 1225994-0101 |  |  |
| 1 | CV06 | 1 | L | 6-9612 22 0008 | Non-return valve | LKC-2-W-ISO-38.0-316L EPDM | 1241273-0101 |  |  |
| 1 | CV11 | 1 | L | 6-9612 22 0008 | Non-return valve | LKC-2-W-ISO-38.0-316L EPDM | 1241273-0101 |  |  |
| 1 | CV12 | 1 | L | 6-9612 22 0009 | Non-return valve | LKC-2-W-ISO-51.0-316L EPDM | 1241273-0101 |  |  |
| 1 | CV13 | 1 | L | 90517-2812 | Non-return valve | 1.5" Sandwich type EPDM | 1224311-0101 |  |  |

| Item No. | Label | No |  | Article No. | Denomination | Type | Doc No | Note | Rev. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | CV44 | 1 | L | 6-9612 22 0008 | Non-return valve | LKC-2-W-ISO-38.0-316L EPDM | 1241273-0101 |  |  |
| 1 | CV66 | 1 | L | 6-9612 22 0007 | Non-return valve | LKC-2-W-ISO-25.0-316L EPDM | 1241273-0101 |  |  |
| 1 | CV67 | 1 | L | 90503-8930 | Non-return valve | Metal Work 0521/0602-1/4" | No doc. |  |  |
| 1 | CV68 | 1 | L | 90503-8930 | Non-return valve | Metal Work 0521/0602-1/4" | No doc. |  |  |
| 1 | DV09 | 1 | L | 90513-8306 | Air vent valve SS | With ball valve DN15 PN10, no fitting Type AB050/R002 | 1212676-0101 |  |  |
| 1 | DV10.2 | 1 | L | 90513-8306 | Air vent valve SS | With ball valve DN15 PN10, no fitting Type AB050/R002 | 1212676-0101 |  |  |
| 1 | DV10 | 1 | L | 6-34144 8035 1 | Micro bubble separator | NONAIR W DN65 adjusted 15 deg. AISI316 According to drawing: 34144-8035 Rev. 01 | No doc. |  |  |
| 1 | DV66.2 | 1 | L | 90513-8306 | Air vent valve SS | With ball valve DN15 PN10, no fitting Type AB050/R002 | 1212676-0101 |  |  |
| 1 | DV66 | 1 | L | 6-34144 8034 1 | Micro bubble separator | NONAIR W DN50 adjusted 15 deg. AISI316 According to drawing: 34144-8034 Rev. 01 | No doc. |  |  |
| 1 | F44 | 1 | L | 6-31801 5044 1 | Strainer | Fig 37 SG DN25 PN40 GGG-40 Mesh=0.8mm | 1241073-0101 |  |  |
| 1 | FB83 | 1 | L | 90511-1954 | Sensor cable | 10m angled M12-connection Without LED | No Doc. |  |  |
| 1 | FB83 | 1 | L | 6-9613 41 8104 | Top unit | IndiTop 8-30V AC/DC PNP With 0.5m cable, M12-5 contact | 1221736-0101 |  |  |
| 1 | FB84 | 1 | L | 90521-6272 | Top control | 8697 2 sw 1 M16 | 3280255-0101 |  |  |
| 1 | FB84 | 1 | L | 90521-1069 | Adapter kit | For 8697 to 50/63/80C actuator | No doc. |  |  |
| 1 | FI09 | 1 | L | 90515-8855 | Flow indicator | H250/RR/M40 2" SMS 1145 PN16 1000-10000l/h | 1229491-0101 |  |  |
| 1 | GS40 | 1 | L | 90510-0270 | Proximity switch | M18x1 L=70mm, SS, IP69K, M12-conn. 10-36VDC, 3-w.PNP/2 w.PNP/NPN Sensing range 0-8mm | 1241250-0101 |  |  |
| 1 | GS40 | 1 | L | 344254-0100 | Sensor cable, EVT005 | Cable length 10 m | No doc. |  |  |
| 1 | HE02 | 1 | L | 6-32360 0090 2 | Insulation | CB76/CB77 31-60 plates | No doc. |  |  |
| 1 | HE02 | 1 | L | 6-32871 0175 4 | Brazed plate heat exchanger | CB110-30L B23 O5 | 1210229-0101 |  |  |
| 1 | LE08 | 1 | L | 6-32249 0310 5 | Seal kit | For bend dial thermometer 12mm | No doc. |  |  |
| 1 | LE08 | 1 | L | 90511-7667 | Connection box | 55x55x37 2 way M20 IP66, gray, polyester | No doc. |  |  |
| 1 | LE08 | 1 | L | 90522-2085 | Level Sensor | UE-VK15 L1=125 L2=925 "Resolution 15mm, pipe 12mm" "PTFE cable 7m, Float type SV" | 1241312-0101 |  |  |
| 1 | LE08 | 1 | L | 6-32010 2248 1 | Nut | Acc.to drawing: 32010-2248 | No doc. |  |  |
| 1 | LS50 | 1 | L | 90522-2130 | Level switch | FTL33 NPT1/2 10-30V DC, 3-w PNP Compact 316L, Ra<0.76um Plug M12, IP 65/67 NEMA 4X | 3283723-0101 |  |  |
| 1 | LS50 | 1 | L | 344254-0100 | Sensor cable, EVT005 | Cable length 10 m | No doc. |  |  |
| 1 | LS51 | 1 | L | 344254-0100 | Sensor cable, EVT005 | Cable length 10 m | No doc. |  |  |
| 1 | LS51 | 1 | L | 90522-2130 | Level switch | FTL33 NPT1/2 10-30V DC, 3-w PNP Compact 316L, Ra<0.76um Plug M12, IP 65/67 NEMA 4X | 3283723-0101 |  |  |
| 1 | LSL66 | 1 | L | 344254-0100 | Sensor cable, EVT005 | Cable length 10 m | No doc. |  |  |
| 1 | LSL66 | 1 | L | 90522-2130 | Level switch | FTL33 NPT1/2 10-30V DC, 3-w PNP Compact 316L, Ra<0.76um Plug M12, IP 65/67 NEMA 4X | 3283723-0101 |  |  |
| 1 | LT10 | 1 | L | 90511-2809 | Level switch | ALVTC Tri-Cl. 2.5" L=850mm 2-w. 326L/316Ti ALVTC2.5- TP43A-VK10-L850-SV52V-SR69 | 1224261-0101 |  |  |
| 1 | M10 | 1 | L | 6-9613 39 4878 | Centrifugal pump | LKH-20/165-DMS-SMS-5.5kW-EPDM is C/SiC 50Hz 230D/400- 415Y 2925rpm ABB IE3 | 1214246-0101 |  |  |
| 1 | M2 | 1 | L | 2589066-0100 | Circumferential Piston Pumps | SCPP2/180 80mm SMS DMS 22kW EPDM SIC/SIC 50Hz Twin wing 93degC Horizontal 50Hz 3x400V Nord IE2 IP55 | 1226903-0101 |  |  |
| 1 | M30 | 1 | L | 90503-1522 | Agitator | Scaba 20D24-TA 0.37kW 750RPM | No doc. | Included in BTD01 |  |
| 1 | M3 | 1 | L | 6-9613 39 4866 | Centrifugal pump | LKH-35/220-SSS-SMS-11.0kW-EPDM is C/SiC 50Hz 230D/400-415Y ABB IE3 | 1214246-0101 |  |  |

| Item No. | Label | No |  | Article No. | Denomination | Type | Doc No | Note | Rev. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | M9 | 1 | L | 90511-6143 | Centrifugal pump | CRN15-3 DN50 3kW 50Hz SiC/SiC EPDM 230-240D/380- 415YV 50Hz Clamp couplings | 1218037-0101 |  |  |
| 1 | MFC10 | 1 | L | 90503-8200 | Constant flow valve | Flow restrictor G 1/2 REG-1200-D 0.5l/mi | 1221586-0101 |  |  |
| 1 | MFC21 | 1 | L | 90503-8200 | Constant flow valve | Flow restrictor G 1/2 REG-1200-D 0.5l/mi | 1221586-0101 |  |  |
| 1 | MFC25 | 1 | L | 90503-8200 | Constant flow valve | Flow restrictor G 1/2 REG-1200-D 0.5l/mi | 1221586-0101 |  |  |
| 1 | OP66 | 1 | T |  | Orifice plate incl in P&F. 5mm |  |  |  |  |
| 1 | PC01 | 1 | L | 90519-8606 | Pressure regulator | FRL-G3/8-10 ISO228/1 1.5-10bar(g) incl. pressure switch, pressure indicato filter 5um | 3283771-0101 |  |  |
| 1 | PC107 | 1 | L | 90503-3190 | Air regulator | REG P ISO228/1-G1/4 0-8bar(G) lockable | 1241164-0101 |  |  |
| 1 | PC67 | 1 | L | 90503-3190 | Air regulator | REG P ISO228/1-G1/4 0-8bar(G) lockable | 1241164-0101 |  |  |
| 1 | PC68 | 1 | L | 90503-3190 | Air regulator | REG P ISO228/1-G1/4 0-8bar(G) lockable | 1241164-0101 |  |  |
| 1 | PC76 | 1 | L | 90503-3190 | Air regulator | REG P ISO228/1-G1/4 0-8bar(G) lockable | 1241164-0101 |  |  |
| 1 | PI09 | 1 | L | 90522-0142 | Pressure gauge | G3/8" A 0-25bar v. 233.50.100 lower mount | 1212430-0101 |  |  |
| 1 | PI10 | 1 | L | 90522-0142 | Pressure gauge | G3/8" A 0-25bar v. 233.50.100 lower mount | 1212430-0101 |  |  |
| 1 | PT44 | 1 | L | 90525-4814 | Pressure transmitter | PTP31B g1/2 0-10bar(g) PNP 4-20mA SS house display Plug M12, IP65/67 NEMA Type 4X | 3288665-0101 |  |  |
| 1 | PT44 | 1 | L | 344254-0100 | Sensor cable, EVT005 | Cable length 10 m | No doc. |  |  |
| 1 | PT66 | 1 | L | 344254-0100 | Sensor cable, EVT005 | Cable length 10 m | No doc. |  |  |
| 1 | PT66 | 1 | L | 90525-4814 | Pressure transmitter | PTP31B g1/2 0-10bar(g) PNP 4-20mA SS house display Plug M12, IP65/67 NEMA Type 4X | 3288665-0101 |  |  |
| 1 | PX74 | 1 | L | 90510-8981 | I/P converter | ITV1050 0-0.9MPa 24V DC 4-20m A/PNP ISO228/1-G3/8 M12 Male 4-pin | 1224084-0101 |  |  |
| 1 | PX74 | 1 | L | 344254-0100 | Sensor cable, EVT005 | Cable length 10 m | No doc. |  |  |
| 1 | QT76 | 1 | L | 90503-4549 | Conductivity meter | CLD132 ISO Cl51 2x4-20mA 24V AC/DC Display SS house w.5m cable Conductivity HART and temperature output | 1218800-0101 |  |  |
| 1 | Sign | 2 | L | 351552-0043 | Warning Sign ISO | Chemical Hazard, 75 x 75 mm | 3286259-0101 |  |  |
| 1 | Sign | 1 | L | 359705-0018 | Warning Sign ISO, Hot Cor.Subst.U H P |  | 3284690-0101 |  |  |
| 1 | Sign | 2 | L | 351552-0004 | Warning Sign ISO | Chemical Hazard, 100 x 100 mm | 3286259-0101 |  |  |
| 1 | Sign | 1 | L | 351554-0005 | Mandatory Action Sign ISO | Wear ear protection | 3286259-0101 |  |  |
| 1 | Sign | 1 | L | 359705-0008 | Warning Sign ISO, Burn Hazard |  | 3284690-0101 |  |  |
| 1 | Sign | 1 | L | 351552-0065 | Warning Sign ISO | Hot Outlet, 50 x 50 mm | 3286259-0101 |  |  |
| 1 | Sign | 1 | L | 90511-0145 | Logotype | Tetra Pak 42x316.26mm, white For stripe 70mm | No doc. |  |  |
| 1 | Stripe | 25 | L | 90511-8563 | Stripe | Blue, height 70mm Supplied on roll length 25m | No doc. |  |  |
| 1 | SX02 | 1 | T |  | Frequency converter |  |  |  |  |
| 1 | SX09 | 1 | T |  | Freqvency converter |  |  |  |  |
| 1 | SX10 | 1 | T |  | Freqvency converter |  |  |  |  |
| 1 | SX30 | 1 | T |  | Freqvency converter |  |  |  |  |
| 1 | TT01 | 1 | L | 344254-0100 | Sensor cable, EVT005 | Cable length 10 m | No doc. |  |  |
| 1 | TT01 | 1 | L | 90510-5633 | Temperature transmitter | TPT12 4-20mA Measuring range 0-160 deg. C | 3283902-0101 |  |  |
| 1 | TT09 | 1 | L | 90510-5633 | Temperature transmitter | TPT12 4-20mA Measuring range 0-160 deg. C | 3283902-0101 |  |  |
| 1 | TT09 | 1 | L | 344254-0100 | Sensor cable, EVT005 | Cable length 10 m | No doc. |  |  |
| 1 | V05 | 1 | L | 6-9612 62 8353 | Butterfly valve | LKB-W-25-316L-EPDM-P85-NC | 1215850-0101 |  |  |
| 1 | V08 | 1 | L | 90512-8694 | Valve connector | Type A, SAC-3P-10,0-PVC/A Cable 10m | No doc. |  |  |

| Item No. | Label | No |  | Article No. | Denomination | Type | Doc No | Note | Rev. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | V08 | 1 | L | 90521-7263 | Regulating valve | 3347/3372-3725 120cm2 W38.0x1.2 NO EN10357-D PN16 1.4409 SS/SS Kvs16.0 | 1224799-0101 |  |  |
| 1 | V107 | 1 | L | 90503-7947 | Seat valve | ZQ/L W38 AISI316L SS/SS Act.108 A/A Giotto Top 2xprox.PNP/2xsol. 3/2 24VDC | 1222092-0101 |  |  |
| 1 | V109 | 1 | L | 6-9612 62 8355 | Butterfly valve | LKB-W-51-316L-EPDM-P85-NC | 1215850-0101 |  |  |
| 1 | V10 | 1 | L | 90512-7413 | Seat valve | 2000/32 G3/8 NC Kvs=2.4 | 3282610-0101 |  |  |
| 1 | V114 | 1 | L | 6-9613 36 0002 | Single seat valve | SSV-W-51-200-NC-EPDM Shut off | 1221121-0101 |  |  |
| 1 | V115 | 1 | L | 6-9612 62 8709 | Butterfly valve | LKB-W-51-316L-EPDM-P85-NO | 1215850-0101 |  |  |
| 1 | V11 | 1 | L | 6-9613 36 3174 | Single seat valve | SSV-W-38-210-NC-EPDM TP Change Over Maintainable actuator | 1221122-0101 |  |  |
| 1 | V13 | 1 | L | 6-9612 62 8354 | Butterfly valve | LKB-W-38-316L-EPDM-P85-NC | 1215850-0101 |  |  |
| 1 | V14 | 1 | L | 6-9613 36 0066 | Single seat valve | SSV-W-38-210-NO-EPDM Change Over | 1221122-0101 |  |  |
| 1 | V15 | 1 | L | 6-9613 36 3184 | Single seat valve | SSV-W-38-210-NO-EPDM TP Change Over Maintainable actuator | 1221122-0101 |  |  |
| 1 | V16 | 1 | L | 6-9613 36 0054 | Single seat valve | SSV-W-51-210-NC-EPDM Change Over | 1221122-0101 |  |  |
| 1 | V17 | 1 | L | 6-9612 62 8355 | Butterfly valve | LKB-W-51-316L-EPDM-P85-NC | 1215850-0101 |  |  |
| 1 | V18 | 1 | L | 6-9612 62 8709 | Butterfly valve | LKB-W-51-316L-EPDM-P85-NO | 1215850-0101 |  |  |
| 1 | V25 | 1 | L | 90512-7413 | Seat valve | 2000/32 G3/8 NC Kvs=2.4 | 3282610-0101 |  |  |
| 1 | V44 | 1 | L | 90512-8694 | Valve connector | Type A, SAC-3P-10,0-PVC/A Cable 10m | No doc. |  |  |
| 1 | V44 | 1 | L | 6-31801 9423 4 | Regulating valve | Samson 3321/3372 120cm2 GG DIN DN32 PN16 Kvs=16 SS/SS NC | 1224796-0101 |  |  |
| 1 | V59 | 1 | L | 6-31801 9975 2 | Seat valve | 2000/63 W38*1.2 NO 16bar(g) Kvs=35 | 1218030-0101 |  |  |
| 1 | V66.2 | 1 | L | 90512-7416 | Seat valve | 2000/32 W17.2*1.6 NC Kvs=2.4 | 3282610-0101 |  |  |
| 1 | V66 | 1 | L | 90512-7416 | Seat valve | 2000/32 W17.2*1.6 NC Kvs=2.4 | 3282610-0101 |  |  |
| 1 | V68 | 1 | L | 6-31880 0235 1 | Back pressure valve | Designed for 8 bar at 175 deg.C Acc.to drawing 6-31880-0235 Rev.01 | 1223553-0101 |  |  |
| 1 | V74 | 1 | L | 6-9612 65 0173 | Constant pressure valve | CPMI-2-W-51-Kvs 23 EPDM PTFE// | 1214498-0101 |  |  |
| 1 | V75 | 1 | L | 6-9613 36 0457 | Single seat valve | SSV-W-38-111-NC-EPDM RA Change Over | 1221124-0101 |  |  |
| 1 | V76 | 1 | L | 6-9613 32 4048 | Single seat valve | SSV-W-38-112-NO-EPDM RA Change Over | 1221124-0101 |  |  |
| 1 | V80 | 1 | L | 6-31801 9971 2 | Seat valve | 2000/63 W25*1.2 NC 11bar(g) Kvs=19 | 1218030-0101 |  |  |
| 1 | V82 | 1 | L | 6-31801 9971 2 | Seat valve | 2000/63 W25*1.2 NC 11bar(g) Kvs=19 | 1218030-0101 |  |  |
| 1 | V83 | 1 | L | 6-9612 49 1817 | Mounting bracket | LKLA 85 for Meca-Inox ball valve For body size DN8-15 | No doc. |  |  |
| 1 | V83 | 1 | L | 6-9612 19 4001 | Actuator | LKLA-T-85-NC | 1215850-0101 |  |  |
| 1 | V83 | 1 | L | 90511-2615 | Ball valve | Manual SMS-W 25x1.2 red. bore DV20+CVMM 3-way | 1224238-0101 |  |  |
| 1 | V84 | 1 | L | 6-31801 9975 1 | Seat valve | 2000/63 W25*1.2 NO 16bar(g) Kvs=19 | 1218030-0101 |  |  |
| 1 | V85.1 | 1 | L | 6-31801 9971 2 | Seat valve | 2000/63 W25*1.2 NC 11bar(g) Kvs=19 | 1218030-0101 |  |  |
| 1 | V85.2 | 1 | L | 6-31801 9971 2 | Seat valve | 2000/63 W25*1.2 NC 11bar(g) Kvs=19 | 1218030-0101 |  |  |
| 1 | V86 | 1 | L | 90518-7758 | Sensor cable | 15m, PVC, 8-pole 1xM12 angled, A-coded | No doc. |  |  |
| 1 | V86 | 1 | L | 90523-6505 | Regulating valve | 8802GD/8694 24VDC 70E Kvs 7.1 NC EN10357-D W25.0.x1.2 316L SS/SS Max. dP 16 bar(g) | 1226927-0101 |  |  |
| 1 | V87 | 1 | L | 90513-5612 | Seat valve | 2000/125 W51*1.2 NC 10bar(g) Kvs 55 | 1218030-0101 |  |  |

| Item No. | Label | No |  | Article No. | Denomination | Type | Doc No | Note | Rev. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 Booster module 3003921-0100 |  |  |  |  |  |  |  |  |  |
| 1 | CR46 | 1 | L | 6-31801 9809 1 | Steam trap | MST21 G1/2" NTS-fill PN25 | 1212764-0101 |  |  |
| 1 | CR49.1 | 1 | L | 6-31801 9809 1 | Steam trap | MST21 G1/2" NTS-fill PN25 | 1212764-0101 |  |  |
| 1 | CR50.1 | 1 | L | 6-31801 9809 1 | Steam trap | MST21 G1/2" NTS-fill PN25 | 1212764-0101 |  |  |
| 1 | CV07 | 1 | L | 6-9612 22 0008 | Non-return valve | LKC-2-W-ISO-38.0-316L EPDM | 1241273-0101 |  |  |
| 1 | CV53 | 1 | L | 90512-6750 | Non-return valve | Aseptomag RSV1.5" TVE Clamp Ra0.8 EPDM | 1219823-0101 |  |  |
| 1 | FB71 | 1 | L | 6-9613 41 9801 | Top unit | ThinkTop Basic 10-30VDC PNP/NPN | 1222265-0101 |  |  |
| 1 | FT04 | 1 | L | 90503-1879 | Magnetic Flow meter | Promag 50H ISO W25.0 0-18000l/h PFA Display 20-55V AC/16-62V DC w.10m cable Pulse signal / 4-20mA HART 0.5% cal. | 1218415-0101 |  |  |
| 1 | GS10 | 1 | L | 90510-0270 | Proximity switch | M18x1 L=70mm, SS, IP69K, M12-conn. 10-36VDC, 3-w.PNP/2 w.PNP/NPN Sensing range 0-8mm | 1241250-0101 |  |  |
| 1 | GS10 | 1 | L | 344254-0100 | Sensor cable, EVT005 | Cable length 10 m | No doc. |  |  |
| 1 | GS11 | 1 | L | 90510-0270 | Proximity switch | M18x1 L=70mm, SS, IP69K, M12-conn. 10-36VDC, 3-w.PNP/2 w.PNP/NPN Sensing range 0-8mm | 1241250-0101 |  |  |
| 1 | GS11 | 1 | L | 344254-0100 | Sensor cable, EVT005 | Cable length 10 m | No doc. |  |  |
| 1 | GS12 | 1 | L | 344254-0100 | Sensor cable, EVT005 | Cable length 10 m | No doc. |  |  |
| 1 | GS12 | 1 | L | 90510-0270 | Proximity switch | M18x1 L=70mm, SS, IP69K, M12-conn. 10-36VDC, 3-w.PNP/2 w.PNP/NPN Sensing range 0-8mm | 1241250-0101 |  |  |
| 1 | GS16 | 1 | L | 90510-0270 | Proximity switch | M18x1 L=70mm, SS, IP69K, M12-conn. 10-36VDC, 3-w.PNP/2 w.PNP/NPN Sensing range 0-8mm | 1241250-0101 |  |  |
| 1 | GS16 | 1 | L | 344254-0100 | Sensor cable, EVT005 | Cable length 10 m | No doc. |  |  |
| 1 | GS20 | 1 | L | 344254-0100 | Sensor cable, EVT005 | Cable length 10 m | No doc. |  |  |
| 1 | GS20 | 1 | L | 90510-0270 | Proximity switch | M18x1 L=70mm, SS, IP69K, M12-conn. 10-36VDC, 3-w.PNP/2 w.PNP/NPN Sensing range 0-8mm | 1241250-0101 |  |  |
| 1 | GS21 | 1 | L | 90510-0270 | Proximity switch | M18x1 L=70mm, SS, IP69K, M12-conn. 10-36VDC, 3-w.PNP/2 w.PNP/NPN Sensing range 0-8mm | 1241250-0101 |  |  |
| 1 | GS21 | 1 | L | 344254-0100 | Sensor cable, EVT005 | Cable length 10 m | No doc. |  |  |
| 1 | M4 | 1 | L | 6-9613 39 4833 | Centrifugal pump | LKHM-114/163-FSS-SMS-15.0kW-EPDM is C/SiC 50Hz 230D/400-415Y ABB IE3 | 1214248-0101 |  |  |
| 1 | PC46 | 1 | L | 6-31801 1768 1 | Reducing valve | BRV2S DN20 ISO7/1-Rp3/4 GGG-40.3 0.14-1.7bar(g) Kvs=2.5 | 1248679-0101 |  |  |
| 1 | PC51A | 1 | L | 90503-3190 | Air regulator | REG P ISO228/1-G1/4 0-8bar(G) lockable | 1241164-0101 |  |  |
| 1 | PC53B | 1 | L | 90503-3190 | Air regulator | REG P ISO228/1-G1/4 0-8bar(G) lockable | 1241164-0101 |  |  |
| 1 | PI03 | 1 | L | 90522-0152 | Pressure gauge | SMS38 0-2.5MPa/0-25bar(g) v. 233.50.100 lower mount 990.19 DN 1 1/2" union nut | 1212430-0101 |  |  |
| 1 | PI07 | 1 | L | 90522-0156 | Pressure gauge | SMS38 0-1MPa/0-10bar(g) h. 233.50.100 lower back mount 990.19 DN 1 1/2" union nut | 1212430-0101 |  |  |
| 1 | PI46 | 1 | L | 90522-0140 | Pressure gauge | G3/8" 0-1MPa/0-10bar(g) v. 233.50.100 lower mount | 1212430-0101 |  |  |
| 1 | PT06 | 1 | L | 90515-5430 | Pressure transmitter | PMP55 SMS38.0 0-10bar(g) 4-20mA , F15 SS house Cable gland M20x1.5 IP66/68 | 1229086-0101 |  |  |
| 1 | PT51 | 1 | L | 90515-5472 | Pressure transmitter | PMP55 SMS38.0 0-37.5bar(g) 4-20mA , F15 SS house Cable gland M20x1.5 IP66/68 | 1229086-0101 |  |  |
| 1 | PT61A | 1 | L | 90515-5430 | Pressure transmitter | PMP55 SMS38.0 0-10bar(g) 4-20mA , F15 SS house Cable gland M20x1.5 IP66/68 | 1229086-0101 |  |  |

| Item No. | Label | No |  | Article No. | Denomination | Type | Doc No | Note | Rev. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | PT61B | 1 | L | 90516-3155 | Pressure transmitter | PMP51 ISO228 G1/2 0-10bar(g) 4-20mA F15 SS house w/o display Cable gland M20x1.5 IP66/68 | 1229086-0101 |  |  |
| 1 | PT64A | 1 | L | 90515-5430 | Pressure transmitter | PMP55 SMS38.0 0-10bar(g) 4-20mA , F15 SS house Cable gland M20x1.5 IP66/68 | 1229086-0101 |  |  |
| 1 | PT64B | 1 | L | 90516-3155 | Pressure transmitter | PMP51 ISO228 G1/2 0-10bar(g) 4-20mA F15 SS house w/o display Cable gland M20x1.5 IP66/68 | 1229086-0101 |  |  |
| 1 | PT67A | 1 | L | 90515-5430 | Pressure transmitter | PMP55 SMS38.0 0-10bar(g) 4-20mA , F15 SS house Cable gland M20x1.5 IP66/68 | 1229086-0101 |  |  |
| 1 | PT67B | 1 | L | 90516-3155 | Pressure transmitter | PMP51 ISO228 G1/2 0-10bar(g) 4-20mA F15 SS house w/o display Cable gland M20x1.5 IP66/68 | 1229086-0101 |  |  |
| 1 | PX78 | 1 | L | 90510-8981 | I/P converter | ITV1050 0-0.9MPa 24V DC 4-20m A/PNP ISO228/1-G3/8 M12 Male 4-pin | 1224084-0101 |  |  |
| 1 | PX78 | 1 | L | 344254-0100 | Sensor cable, EVT005 | Cable length 10 m | No doc. |  |  |
| 1 | Sign | 1 | L | 6-34144 8572 1 | Sign | Open route Self adhesive plastic film According to drawing: 34144-8572_02 | 3284690-0101 |  |  |
| 1 | Sign | 1 | L | 6-34144 8571 1 | Sign | Self adhesive plastic film Air pressure for V51 According to drawing: 34144-8571_01 | 3284690-0101 |  |  |
| 1 | Stripe | 20 | L | 90511-8563 | Stripe | Blue, height 70mm Supplied on roll length 25m | No doc. |  |  |
| 1 | SX04 | 1 | T |  | Frequency converter |  |  |  |  |
| 1 | TT02 | 1 | L | 90510-5633 | Temperature transmitter | TPT12 4-20mA Measuring range 0-160 deg. C | 3283902-0101 |  |  |
| 1 | TT02 | 1 | L | 344254-0100 | Sensor cable, EVT005 | Cable length 10 m | No doc. |  |  |
| 1 | TT03 | 1 | L | 344254-0100 | Sensor cable, EVT005 | Cable length 10 m | No doc. |  |  |
| 1 | TT03 | 1 | L | 90510-5633 | Temperature transmitter | TPT12 4-20mA Measuring range 0-160 deg. C | 3283902-0101 |  |  |
| 1 | TT06 | 1 | L | 344254-0100 | Sensor cable, EVT005 | Cable length 10 m | No doc. |  |  |
| 1 | TT06 | 1 | L | 90510-5633 | Temperature transmitter | TPT12 4-20mA Measuring range 0-160 deg. C | 3283902-0101 |  |  |
| 1 | TT08 | 1 | L | 344254-0100 | Sensor cable, EVT005 | Cable length 10 m | No doc. |  |  |
| 1 | TT08 | 1 | L | 90510-5633 | Temperature transmitter | TPT12 4-20mA Measuring range 0-160 deg. C | 3283902-0101 |  |  |
| 1 | TT26 | 1 | L | 90510-5633 | Temperature transmitter | TPT12 4-20mA Measuring range 0-160 deg. C | 3283902-0101 |  |  |
| 1 | TT26 | 1 | L | 344254-0150 | Sensor cable |  | No doc. |  |  |
| 1 | TT51 | 1 | L | 344254-0100 | Sensor cable, EVT005 | Cable length 10 m | No doc. |  |  |
| 1 | TT51 | 1 | L | 90510-5633 | Temperature transmitter | TPT12 4-20mA Measuring range 0-160 deg. C | 3283902-0101 |  |  |
| 1 | TT63 | 1 | L | 90510-5633 | Temperature transmitter | TPT12 4-20mA Measuring range 0-160 deg. C | 3283902-0101 |  |  |
| 1 | TT63 | 1 | L | 344254-0150 | Sensor cable |  | No doc. |  |  |
| 1 | TT64 | 1 | L | 344254-0150 | Sensor cable |  | No doc. |  |  |
| 1 | TT64 | 1 | L | 90510-5633 | Temperature transmitter | TPT12 4-20mA Measuring range 0-160 deg. C | 3283902-0101 |  |  |
| 1 | TT67 | 1 | L | 344254-0150 | Sensor cable |  | No doc. |  |  |
| 1 | TT67 | 1 | L | 90510-5633 | Temperature transmitter | TPT12 4-20mA Measuring range 0-160 deg. C | 3283902-0101 |  |  |
| 1 | TT71 REF | 1 | T |  | Plug for PT100 |  |  |  |  |
| 1 | TT71 | 1 | L | 90510-5633 | Temperature transmitter | TPT12 4-20mA Measuring range 0-160 deg. C | 3283902-0101 |  |  |
| 1 | TT71 | 1 | L | 344254-0100 | Sensor cable, EVT005 | Cable length 10 m | No doc. |  |  |
| 1 | V26 | 1 | L | 90518-7758 | Sensor cable | 15m, PVC, 8-pole 1xM12 angled, A-coded | No doc. |  |  |
| 1 | V26 | 1 | L | 90513-3131 | Regulating valve | 8802YG/90E/8694 W DN40 NC SS/PTFE Kvs 34 SMS 3008 38,0 | 1226927-0101 |  |  |
| 1 | V27 | 1 | L | 6-9613 36 0066 | Single seat valve | SSV-W-38-210-NO-EPDM Change Over | 1221122-0101 |  |  |
| 1 | V28 | 1 | L | 6-9613 36 0066 | Single seat valve | SSV-W-38-210-NO-EPDM Change Over | 1221122-0101 |  |  |

| Item No. | Label | No |  | Article No. | Denomination | Type | Doc No | Note | Rev. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | V46 | 1 | L | 6-31801 9971 2 | Seat valve | 2000/63 W25*1.2 NC 11bar(g) Kvs=19 | 1218030-0101 |  |  |
| 1 | V49 | 1 | L | 90512-8694 | Valve connector | Type A, SAC-3P-10,0-PVC/A Cable 10m | No doc. |  |  |
| 1 | V49 | 1 | L | 90521-7203 | Regulating valve | 3321/3372-3725 120cm2 DN20 Kvs6.3 NC EN1092 PN16 EN- JL1040 SS/SS Pressure gauge | 1224797-0101 |  |  |
| 1 | V50 | 1 | L | 90521-7203 | Regulating valve | 3321/3372-3725 120cm2 DN20 Kvs6.3 NC EN1092 PN16 EN- JL1040 SS/SS Pressure gauge | 1224797-0101 |  |  |
| 1 | V50 | 1 | L | 90512-8694 | Valve connector | Type A, SAC-3P-10,0-PVC/A Cable 10m | No doc. |  |  |
| 1 | V51 | 1 | L | 6-31801 5878 2 | Seat valve | YQ/T W38 AISI316L SS/SS Act.108 A/A Empty Giotto Top | 1222092-0101 |  |  |
| 1 | V52 | 1 | L | 6-9613 36 0040 | Single seat valve | SSV-W-38-300-NO-EPDM Shut off | 1221121-0101 |  |  |
| 1 | V53 | 1 | L | 90510-1963 | Change over valve | ZQ/LL W38 AISI316L SS/PTFE Act. 108 A/A, Empty Giotto Top | 1222092-0101 |  |  |
| 1 | V63 | 1 | L | 90518-7758 | Sensor cable | 15m, PVC, 8-pole 1xM12 angled, A-coded | No doc. |  |  |
| 1 | V63 | 1 | L | 90513-3139 | Regulating valve | 8802YG/70E/8694 W DN32 NC SS/PTFE Kvs 23 SMS 3008 38,0 | 1226927-0101 |  |  |
| 1 | V64 | 1 | L | 90513-3138 | Regulating valve | 8802YG/70E/8694 W DN20 NC SS/PTFE Kvs 10 SMS 3008 25,0 | 1226927-0101 |  |  |
| 1 | V64 | 1 | L | 90518-7758 | Sensor cable | 15m, PVC, 8-pole 1xM12 angled, A-coded | No doc. |  |  |
| 1 | V67 | 1 | L | 90513-3138 | Regulating valve | 8802YG/70E/8694 W DN20 NC SS/PTFE Kvs 10 SMS 3008 25,0 | 1226927-0101 |  |  |
| 1 | V67 | 1 | L | 90518-7758 | Sensor cable | 15m, PVC, 8-pole 1xM12 angled, A-coded | No doc. |  |  |
| 1 | V71 | 1 | L | 6-9613 36 0577 | Single seat valve | SSV-W-38-220-NC-EPDM as. Change Over | 1222462-0101 |  |  |
| 1 | V778 | 1 | L | 6-9613 36 0506 | Single seat valve | SSV-W-51-200-NC-EPDM as. Shut off | 1222444-0101 |  |  |
| 1 | V78 | 1 | L | 6-32422 0302 1 | Expansion valve | AISI 316 APC W ISO 51 Kvs=14 | 1215611-0101 |  |  |
| 1 | V78 | 1 | L | 6-32422 0724 1 | Protection cover | For APC dia51mm With hinge and pipe holder, EN 1.4301 Acc. to dwg:32422-0724 Rev.01 | No doc. |  |  |
| 1 | V90 | 1 | L | 90510-1963 | Change over valve | ZQ/LL W38 AISI316L SS/PTFE Act. 108 A/A, Empty Giotto Top | 1222092-0101 |  |  |
| 1 | V91 | 1 | L | 6-9613 36 0040 | Single seat valve | SSV-W-38-300-NO-EPDM Shut off | 1221121-0101 |  |  |
| 1 | V92 | 1 | L | 6-9613 36 0053 | Single seat valve | SSV-W-38-210-NC-EPDM Change Over | 1221122-0101 |  |  |
| 1 | V93 | 1 | L | 90521-5830 | Seat valve | ZQ/T W38 AISI316L SS/PTFE Act.108 A/A Empty Giotto Top | 1222092-0101 |  |  |
| 1 | V96 | 1 | L | 90503-4671 | Change over valve | ZQ/LT W38 AISI316L SS/PTFE Act.108 A/A Empty Giotto Top | 1222092-0101 |  |  |
| 1 | V97 | 1 | L | 90512-8694 | Valve connector | Type A, SAC-3P-10,0-PVC/A Cable 10m | No doc. |  |  |
| 1 | V97 | 1 | L | 90521-7263 | Regulating valve | 3347/3372-3725 120cm2 W38.0x1.2 NO EN10357-D PN16 1.4409 SS/SS Kvs16.0 | 1224799-0101 |  |  |
| 1 | V98 | 1 | L | 6-9613 36 0529 | Single seat valve | SSV-W-38-300-NC-EPDM as. Shut off | 1222444-0101 |  |  |

| Qty Level-0 | Qty Level-1 | Article/Doc. No | Description from SAP | Name | Notes | Position(s) | Rev |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 6-164835312 | Frame complete triple 4 sections 6m | Frame Complete,C, Triple, 4 modules,6m | - | Frame |  |
| 122 |  | 6-164836301 | Screw MRT M6x20 A2/A4 | Screw, Frame | - | Frame |  |
| 22 |  | 6-164835401 | Cover 2,7m for 6m frame w plastic ends | Cover,C,6m | - | Frame |  |
| 216 |  | 6-164832402 | Shell support C85 | Shell Support, C85 | - |  |  |
| 53 |  | 6-164832511 | Shell C85 6m 304 | Shell, C85,1.4307/304L,6m | - | I6,H6,H5,I5,I1,H1,G1,G2,F2,E2,D2,D3,D4, C4,C3,C2,A1,A2,A3,A4,A5,B5,B6,A6,A9,I2 ,I8,A12,B12,C12,D12,D11,D10,D9,D8,C8,C 7,D7,E7,F7,G7,I9,H9,G9,G10,H10,I10,I11,H 11,H12,I12,G12,E9 |  |
| 26 |  | 6-99035374 | O-ring 75.57x5.33 EPDM E8502 | O-ring,Union,Shell,d76 | - | I4,I6,F1,I1,E1,C2,A1,A2,A3,A4,A5,A6,A9, A9,I2,I2,I8,A12,G7,I10,I11,I12,G12,G12,E8, E9 |  |
| 212 |  | 6-99035720 | O-ring 82.14x3.53 FKM V8T41 Coated | O-ring, C85,Shell | - | I6,H6,H5,I5,I1,H1,G1,G2,F2,E2,D2,D3,D4, C4,C3,C2,A1,A2,A3,A4,A5,B5,B6,A6,A9,I2 ,I8,A12,B12,C12,D12,D11,D10,D9,D8,C8,C 7,D7,E7,F7,G7,I9,H9,G9,G10,H10,I10,I11,H 11,H12,I12,G12,E9 |  |
| 5 |  | 6-164833807 | Shell Connection 0° C85 316L, SMS | Shell Connection,C85,SMS,0 degr.,1.4404/316L | - | C2,A9,I8,G7,E9 |  |
| 4 |  | 6-164833903 | Shell Return C85 6m 316L, SMS | Shell return, C85,6m | - | I4,F1,E1,E8 |  |
| 17 |  | 6-164833707 | Shell Connection 90° C85 316L, SMS | Shell Connection,C85,SMS,90 degr.,1.4404/316L | - | I6,I1,A1,A2,A3,A4,A5,A6,A9,I2,I2,A12,I10, I11,I12,G12,G12 |  |
| 40 |  | 6-164833607 | Shell Inter Connection C85 316L | Shell Interconnection, C85,1.4404/316L | - | I9-I8,I6-H6,H6-H5,H5-I5,I1-H1,H1-G1,G1- G2,G2-F2,E2-D2,D2-D3,D3-D4,D4-C4,C4- C3,C3-C2,A1-A2,A3-A4,A5-B5,B5-B6,B6-A6,A12-B12,B12-C12,C12-D12,D12-D11,D11-D10,D10-D9,D9-D8,D8-C8,C8-C7,C7-D7,D7-E7,E7-F7,F7-G7,I9-H9,H9-G9,G9-G10,G10-H10,H10-I10,I11-H11,H11-H12,H12-I12 |  |
| 8 |  | 6-164742402 | W Tube insert C-MT 85/4x20-C 6m 316 FT | Tube Insert, C85,4x20C-6,Pulp,1.4404/316L | Spare part number: 6-164763202 | I6,H6,H5,I5,I11,H11,H12,I12 |  |

| Qty Level-0 | Qty Level-1 | Article/Doc. No | Description from SAP | Name | Notes | Position(s) | Rev |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 40 |  | 6-164742401 | W Tube insert C-MT 85/4x20-S 6m 316 FT | Tube Insert, C85,4x20S-6,Pulp,1.4404/316L | Spare part number: 6-164763201 | I1,H1,G1,G2,F2,E2,D2,D3,D4,C4,C3,C2,A1, A2,A3,A4,A5,B5,B6,A6,A12,B12,C12,D12, D11,D10,D9,D8,C8,C7,D7,E7,F7,G7,I9,H9, G9,G10,H10,I10 |  |
| 4 |  | 6-164739601 | W Tube insert C-MT 85/12x16-S 6m 316 | Tube Insert, C85,12x16S-6,1.4404/316L | Spare part number: 6-164763001 | A9,I8,G12,E9 |  |
| 1 |  | 6-164742901 | W Tube insert C-MT 85/1x51-S 6m 316 | Tube Insert, C85,1x51S-6,1.4404/316L | Spare part number: 6-164763601 | I2 |  |
| 212 |  | 6-99035718 | O-ring 69.44x3.53 FKM V8T41 Coated | O-ring, C85, Insert | - | I6,H6,H5,I5,I1,H1,G1,G2,F2,E2,D2,D3,D4, C4,C3,C2,A1,A2,A3,A4,A5,B5,B6,A6,A9,I2 ,I8,A12,B12,C12,D12,D11,D10,D9,D8,C8,C 7,D7,E7,F7,G7,I9,H9,G9,G10,H10,I10,I11,H 11,H12,I12,G12,E9 |  |
| 2 |  | 6-9612402604 | Seal ring EPDM LKB-F DIN | Product Seal,C85,Monotube,EPDM | - | I2 |  |
| 41 |  | 6-164817601 | Bend C-MT 85 Ellipse 316L | Return Bend, C85,Elliptic,Multitube,1.4404/316L | - | A2-A3,A4-A5,I6-H6,H6-H5,H5-I5,I1- H1,H1-G1,G1-G2,G2-F2,E2-D2,D2-D3,D3- D4,D4-C4,C4-C3,C3-C2,A1-A2,A3-A4,A5-B5,B5-B6,B6-A6,A12-B12,B12-C12,C12-D12,D12-D11,D11-D10,D10-D9,D9-D8,D8-C8,C8-C7,C7-D7,D7-E7,E7-F7,F7-G7,I9-H9,H9-G9,G9-G10,G10-H10,H10-I10,I11-H11,H11-H12,H12-I12 |  |
| 16 |  | 6-164761201 | Flange C-MT85 Ø51x1.5 Multi 316L | Product flange, C85,Multitube,1.4404/316L | - | I6,I1,E2,A1,,,A9,,A12,I9,I11,,E9 |  |
| 1 |  | 6-164761101 | Flange C-MT85 Ø51x1.5 Mono 316L | Product flange, C85,Monotube,1.4404/316L | - |  |  |
| 2 |  | 6-164751301 | Eccentric Reducer C-MT85 | Eccentric Reducer,C85,Drainable,Multitube | - | A9,E9 |  |
| 4 |  | 6-164835601 | Tube Return C85 c/c140 6m multi, SMS | Tube Return, C85,Close,Multitube,6m | - | C1,H8,G8,G11 |  |
| 1 |  | 6-164835603 | Tube Return C85 c/c140 6m mono, SMS | Tube Return, C85,Close,Monotube,6m | - | H2 |  |
| 5 |  | 6-99035338 | O-ring 50.39x3.53 EPDM E8502 | O-ring,Union,Tube Return,d51 | - | C1,H2,H8,G8,G11 |  |
| 84 |  | 6-164837613 | Assembly kit C85 elliptic/drainable | Assembly Kit,C85,Elliptic/drainable | - | A9,,,,E9 |  |
|  | 1 | 6-164760003 | Split ring C-MT85 | Split ring C-MT85 |  |  |  |
|  | 4 | 6-99030913 | Screw ISO 4017 A2-70 M6S M10x40 H | Screw ISO 4017 A2-70 M6S M10x40 H |  |  |  |
|  | 1 | 6-99032224 | O-ring 107.54x3.53 AL113 3030-70 EPDM | O-ring 107.54x3.53 AL113 3030-70 EPDM |  |  |  |

| Qty Level-0 | Qty Level-1 | Article/Doc. No | Description from SAP | Name | Notes | Position(s) | Rev |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 22 |  | 6-164837611 | Assembly kit C85 | Assembly Kit,C85 | - | I6,H6,H5,I5,I1,H1,G1,G2,F2,C1,E2,D2,D3, D4,C4,C3,C2,A1,A2,A3,A4,A5,B5,B6,A6,A 9,H2,I2,H8,I8,G8,A12,B12,C12,D12,D11,D1 0,D9,D8,C8,C7,D7,E7,F7,G7,I9,H9,G9,G10, H10,I10,I11,H11,H12,I12,G11,G12,E9 |  |
|  | 1 | 6-164760003 | Split ring C-MT85 | Split ring C-MT85 |  |  |  |
|  | 4 | 6-99030912 | Screw ISO 4017 A2-70 M6S M10x30 H | Screw ISO 4017 A2-70 M6S M10x30 H |  |  |  |
|  | 1 | 6-99032224 | O-ring 107.54x3.53 AL113 3030-70 EPDM | O-ring 107.54x3.53 AL113 3030-70 EPDM |  |  |  |
| 104 |  | 6-164827501 | Seal ring C/MT85 multitube E8502 | Product Seal,C85,Multitube,EPDM | - | I6,H6,H5,I5,I1,H1,G1,G2,F2,E2,D2,D3,D4, C4,C3,C2,A1,A2,A3,A4,A5,B5,B6,A6,A9,I8 ,A12,B12,C12,D12,D11,D10,D9,D8,C8,C7, D7,E7,F7,G7,I9,H9,G9,G10,H10,I10,I11,H1 1,H12,I12,G12,E9 |  |
| 8 |  | 6-164836003 | HT Single Ø51 6m 316L, SMS | Holding Tubes, C85/108,Single,1.4404/316L,6m,d51 | - | F3,G3,G4,F4,E4,E3,F5,G5,G6,F6,E6,E5 |  |
| 1 |  | 6-164836109 | HT Double Ø51 c/c140 red 316L, SMS | Holding Tubes, C85/108,Double | 1.4404/316L,Reduced, d51,X=4094 | F3,G3,G4,F4,E4,E3 |  |
| 1 |  | 6-164836109 | HT Double Ø51 c/c140 red 316L, SMS | Holding Tubes, C85/108,Double | 1.4404/316L,Reduced, d51,X=4079 | F5,G5,G6,F6,E6,E5 |  |
| 8 |  | 6-164837503 | Bend HT Ø51 c/c140 316L, SMS | Return Bend HT, C85/C108, SMS,1.4404/316L,d51 | - | F3-G3,G3-G4,G4-F4,F4-E4,E4-E3,F5- G5,G5-G6,G6-F6,F6-E6,E6-E5 |  |
| 1 |  | 6-164806102 | Special tool 51 L=240, SMS | Special Tool,d51 | - |  |  |
| 20 |  | 6-99035338 | O-ring 50.39x3.53 EPDM E8502 | O-ring,Union,Holding tube,d51 | - | F3,G3,G4,F4,E4,E3,F3,G3,G4,F4,E4,E3,F5, G5,G6,F6,E6,E5,F5,G5,G6,F6,E6,E5 |  |
| 6 |  | 6-99030508 | Tube holder 85/51, c/c=140 | Tube Holder, Tube-HC,C85,d51 | - |  |  |
| 3 |  | 6-99030521 | Tube holder 51/51, c/c=140 | Tube Holder, HC-HC,C85,d51 | - |  |  |
| 94 |  | 6-164837703 | Tube support w Spring C85/53 | Tube Support, C85,d53 | - | C1,H2,H8,G8,G11,F3,G3,G4,F4,E4,E3,F5,G 5,G6,F6,E6,E5 |  |
| 72 |  | 6-164833414 | Shell support blind C85 | Blind, C85 | - | B1,B2,B3,B4,C5,C6,A7,B7,A8,B8,B9,C9,A 10,B10,C10,A11,B11,C11,D1,E3,E4,D5,E5, D6,E6,F8,F9,E10,F10,E11,F11,E12,F12,H3,I 3,H4,H7,I7 |  |
| 2 |  | 6-164837901 | Template Tetra Spiraflo C | Templates | - |  |  |

| Qty Level-0 | Qty Level-1 | Article/Doc. No | Description from SAP | Name | Notes | Position(s) | Rev |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 6-164834901 | Commisioning kit C85 | Commissioning kit, C85 | - |  |  |
| 2 |  | 6-164900901 | Top bar cover support for 6m frame | Top length bar cover support | - |  |  |
| 4 |  | 6-164904301 | Bracket for top bar | Bracket,Insulation | - |  |  |
| 1 |  | 1183366-2_A_01 |  | Isometric and Layout drawing,C85 |  |  |  |
| 1 |  | 6-164834900 |  | Assembly & spare part drawing,C85 | - |  |  |
| 1 |  | 6-164834200 |  | Pressure vessel drawing,C85,PED | - |  |  |

| Unit Weight & Volume | Unit Size |
| --- | --- |
| Product Volume 501 litre | W = 1440 mm |
| Net Weight 2978 kg | H = 2040 mm |
| Gross Weight - | L = 6370 mm |
| Filled Weight 4704 kg |  |

| Type | Manuf. No | Qty | Max press. Shell kpa | Max press. Tube kpa | Max temp. Shell °C | Max temp. Tube °C | Volume Shell litre | Volume Tube litre | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CMP 85/4x20C-6 | 1183366-1-8 | 8 | 1600 | 4000 | 180 | 180 | 23,6 | 6 | 1.4404/316L |
| CMP 85/4x20S-6 | 1183366-9-48 | 40 | 1600 | 4000 | 180 | 180 | 23,6 | 6 | 1.4404/316L |
| CM 85/12x16S-6 | 1183366-49-52 | 4 | 1600 | 4000 | 180 | 180 | 16,9 | 11,6 | 1.4404/316L |
| CD 85/1x51S-6 | 1183366-53-53 | 1 | 1600 | 4000 | 180 | 180 | 19 | 10,6 | 1.4404/316L |